47698

Proposed Rules 	Federal Register

Vol. 85, No. 152

Thursday, August 6, 2020

This section of the FEDERAL REGISTER	•Federal eRulemaking Portal: Go to 	proposal, explain the reason for any

contains notices to the public of the proposed 	https://www.regulations.gov. Follow the 	recommended change, and include

issuance of rules and regulations. The 	instructions for submitting comments. 	supporting data. To ensure the docket

purpose of these notices is to give interested	•Fax: 202–493–2251. 	does not contain duplicate comments,

persons an opportunity to participate in the	•Mail: U.S. Department of 	commenters should submit only one

rule making prior to the adoption of the final

Transportation, Docket Operations, M– 	copy of the comments. Send your

rules.

30, West Building Ground Floor, Room 	comments to an address listed under the

W12–140, 1200 New Jersey Avenue SE, 	ADDRESSES section. Include ‘‘Docket No.

DEPARTMENT OF TRANSPORTATION 	Washington, DC 20590. 	FAA–2020–0686; Product Identifier

•Hand Delivery: Deliver to Mail 	2019–NM–035–AD’’ at the beginning of

Federal Aviation Administration 	address above between 9 a.m. and 5 	your comments.

p.m., Monday through Friday, except 	Except for Confidential Business

14 CFR Part 39 	Federal holidays. 	Information (CBI) as described in the

For Boeing service information 	following paragraph, and other

[Docket No. FAA–2020–0686; Product 	identified in this NPRM, contact Boeing 	information as described in 14 CFR

Identifier 2019–NM–035–AD] 	Commercial Airplanes, Attention: 	11.35, the FAA will post all comments

RIN 2120–AA64 	Contractual & Data Services (C&DS), 	received, without change, as well as a

2600 Westminster Blvd., MC 110–SK57, 	report summarizing each substantive

Airworthiness Directives; The Boeing 	Seal Beach, CA 90740–5600; telephone 	public contact with FAA personnel

Company Airplanes 	562–797–1717; internet https:// 	concerning this proposed rulemaking.

www.myboeingfleet.com. You may view 	Before acting on this proposal, the FAA

AGENCY: Federal Aviation 	this referenced service information at 	will consider all comments received by

Administration (FAA), DOT. 	the FAA, Airworthiness Products 	the closing date for comments. The FAA

ACTION: Notice of proposed rulemaking 	Section, Operational Safety Branch, 	will consider comments filed after the

(NPRM). 	2200 South 216th St., Des Moines, WA. 	comment period has closed if it is

For information on the availability of 	possible to do so without incurring

SUMMARY: The FAA proposes to 	this material at the FAA, call 206–231– 	expense or delay. The FAA may change

supersede Airworthiness Directive (AD) 	3195. It is also available in the Docket 	this NPRM because of those comments.

2018–23–51, which applies to all The 	for this rulemaking, which may be 	Confidential Business Information (CBI)

Boeing Company Model 737–8 and 737– 	found on the internet at https://

9 (737 MAX) airplanes. Since AD 2018– 	www.regulations.gov by searching for 	CBI is commercial or financial

23–51 was issued, the agency has 	and locating Docket No. FAA–2020– 	information that is both customarily and

determined that final corrective action 	0686. 	actually treated as private by its owner.

is necessary to address the unsafe 	Under the Freedom of Information Act

condition. This proposed AD would 	Examining the AD Docket 	(FOIA) (5 U.S.C. 552), CBI is exempt

require installing new flight control 	You may examine the AD docket on 	from public disclosure. If comments

computer (FCC) software, revising the 	the internet at https:// 	responsive to this NPRM contain

existing Airplane Flight Manual (AFM) 	www.regulations.gov by searching for 	commercial or financial information

to incorporate new and revised 	and locating Docket No. FAA–2020– 	that is customarily treated as private,

flightcrew procedures, installing new 	0686; or in person at Docket Operations 	that you actually treat as private, and

MAX display system (MDS) software, 	between 9 a.m. and 5 p.m., Monday 	that is relevant or responsive to this

changing the horizontal stabilizer trim 	through Friday, except Federal holidays. 	NPRM, it is important that you clearly

wire routing installations, completing 	The AD docket contains this NPRM, any 	designate the submitted comments as

an angle of attack sensor system test, 	comments received, and other 	CBI. Please mark each page of your

and performing an operational readiness 	information. The street address for 	submission containing CBI as

flight. This proposed AD would also 	Docket Operations is listed above. 	‘‘PROPIN.’’ The FAA will treat such

apply to a narrower set of airplanes than 	Comments will be available in the AD 		marked submissions as confidential

the superseded AD, and allow operation 	docket shortly after receipt. 		under the FOIA, and they will not be

(dispatch) of an airplane with certain

inoperative systems only if certain

provisions are incorporated in the

operator’s existing FAA-approved

minimum equipment list (MEL). The

FAA is proposing this AD to address the

unsafe condition on these products. 
	FORFURTHERINFORMATIONCONTACT: Ian

Won, Manager, Seattle ACO Branch,

FAA, 2200 South 216th St., Des Moines,

WA 98198; phone and fax: 206–231–

3500; email: 9-FAA-SACO-AD-Inquiry@

faa.gov.

placed in the public docket of this

NPRM. Submissions containing CBI

should be sent to the person identified

in the FORFURTHERINFORMATION

CONTACT section. Any commentary that

the FAA receives which is not

specifically designated as CBI will be

SUPPLEMENTARYINFORMATION: 	placed in the public docket for this

jbell on DSKJLSW7X2PROD with PROPOSALS 
	DATES: The FAA must receive comments

on this proposed AD by September 21, 	Comments Invited 	rulemaking.

2020. 	The FAA invites you to participate in 	Background

ADDRESSES: You may send comments, 	this rulemaking by submitting written 	On October 29, 2018, a Boeing Model

using the procedures found in 14 CFR 	comments, data, or views about this 	737–8 airplane operated by Lion Air

11.43 and 11.45, by any of the following 	proposal. The most helpful comments 	(Lion Air Flight 610) was involved in an

methods: 	reference a specific portion of the 	accident after takeoff from Soekarno-

VerDate Sep<11>2014 	16:21 Aug 05, 2020 	Jkt 250001 	PO 00000 	Frm 00001 	Fmt 4702 	Sfmt 4702 	E:\FR\FM\06AUP1.SGM 	06AUP1

Federal Register / Vol. 85, No. 152 / Thursday, August 6, 2020 / Proposed Rules 47699

Hatta International Airport in Jakarta, 	altitude disagree alert,5and may affect 	and the Ethiopian Civil Aviation

Indonesia, resulting in 189 fatalities. 	the flightcrew’s ability to accomplish 	Authority (ECAA).

Investigation of the accident has been 	continued safe flight and landing. 	The data from the flight data

completed by the Indonesian authorities 	On November 7, 2018, the FAA issued 	recorders, as summarized in reports7of

(Komite Nasional Keselamatan 	Emergency AD 2018–23–51 as an 	the Ethiopian Airlines Flight 302

Transportasi (KNKT)) with assistance 	interim corrective action.6The FAA 	accident and the Lion Air Flight 610

from the National Transportation Safety 	sent Emergency AD 2018–23–51 to all 	accident, indicated that if a single

Board (NTSB) and the FAA of the 	known U.S. owners and operators of 	erroneously high AOA sensor input is

United States, the manufacturer, and the 	Boeing Model 737 MAX airplanes to 	received by the flight control system,

operator. Reports1from the accident 	require revising certificate limitations 	the maneuvering characteristics

investigation indicate that the airplane’s 	and operating procedures of the AFM to 	augmentation system (MCAS)8can

flight control system2generated 	provide the flightcrew with runaway 	command repeated airplane nose-down

repeated airplane nose-down horizontal 	horizontal stabilizer trim procedures to 	trim of the horizontal stabilizer. This

stabilizer trim3commands contributing 	follow under certain conditions. The 	unsafe condition, if not addressed,

to the accident. 	FAA sent Emergency AD 2018–23–51 to 	could cause the flightcrew to have

Following the Lion Air Flight 610 	all affected civil aviation authorities 	difficulty controlling the airplane, and

accident on October 29, 2018, data from 	(CAAs) at the same time. AD 2018–23– 	lead to excessive airplane nose-down

the flight data recorder, which is 	51, Amendment 39–19512 (83 FR 	attitude, significant altitude loss, and

contained in the Indonesian accident 	62697, December 6, 2018; corrected 	impact with terrain.

report (http://knkt.dephub.go.id/knkt/ 	December 11, 2018 (83 FR 63561)), was 	To address the unsafe condition, the

ntsc_aviation/baru/2018%20- 	published in the Federal Register as an 	FAA proposes to require four design

%20035%20-%20PK- 	amendment to 14 CFR 39.13. 	changes: (1) Installing updated flight

On March 10, 2019, a Boeing Model 	control software (with new control laws)

LQP%20Final%20Report.pdf), indicated

737–8 airplane operated by Ethiopian 	for the FCC operational program

that a single erroneously high angle of

Airlines (Ethiopian Airlines Flight 302) 	software (OPS), (2) installing updated

attack (AOA) sensor4input to the flight

was involved in an accident after takeoff 	MDS display processing computer

control system while the flaps are

from Addis Ababa Bole International 	(DPC) software to generate an AOA

retracted can cause repeated airplane

Airport in Addis Ababa, Ethiopia, 	disagree alert,9(3) revising certain AFM

nose-down trim of the horizontal

resulting in 157 fatalities. The accident 	flightcrew operating procedures, and (4)

stabilizer and multiple flightdeck

is under investigation by the Ethiopian 	changing the routing of horizontal

effects.

Accident Investigation Bureau (EAIB) 	stabilizer trim wires. The first design

These effects include stall warning 	with assistance from the NTSB and the 	change is intended to prevent erroneous

activation, airspeed disagree alert, and 	FAA of the United States, the French 	MCAS activation. The second design

Bureau of Enquiry and Analysis for 	change alerts the pilots that the

1 Preliminary KNKT.18.10.35.04 Aircraft 	Civil Aviation Safety (BEA), the 	airplane’s two AOA sensors are

Accident Investigation Report, dated November

2018, and Final KNKT.18.10.35.04 Aircraft 	European Union Aviation Safety Agency 	disagreeing by a certain amount

Accident Investigation Report, dated October 2019, 	(EASA), the manufacturer, the operator, 	indicating a potential AOA sensor

can be found in the AD docket. 	failure. The third design change is

2 The flight control system for 737 MAX airplanes 	5 Stall warning indication is the activation of the 	intended to ensure that the flightcrew

includes two flight control computers, FCC A and 	stick shaker and other warnings. An airspeed 	has the means to recognize and respond

FCC B, which process inputs from the pilots and

aircraft sensors to move the airplane’s control 
	disagree alert, or ‘‘IAS (indicated airspeed)

DISAGREE’’ on the 737 MAX, is a visual alert on 
	to erroneous stabilizer movement and

surfaces. 	the airplane’s primary flight displays (PFDs) that 	the effects of a potential AOA sensor

3 An airplane’s nose-up or nose-down attitude is 	the airspeed displayed on the captain’s and first 	failure. The fourth design change is

known as its ‘‘pitch attitude.’’ On the 737 MAX, the 	officer’s PFDs, as sensed by the pitot tubes on either 	intended to restore compliance with the

airplane’s pitch attitude is primarily controlled by 	side of the airplane, disagree by more than 5 knots 	FAA’s latest wire separation safety

a combination of two movable surfaces on the tail 	for more than 5 seconds. An altitude disagree alert,

of the airplane: The horizontal stabilizer, which is 	or ‘‘ALT (altitude) DISAGREE’’ on the 737 MAX, is 	standards.

controlled by electric and manual (pilot) trim 	a visual alert on the PFDs that the altitude, as 	In addition to these four design

inputs, and the elevator, which is controlled by 	sensed by the static ports on either side of the 	changes, the FAA also proposes to

moving the control columns. ‘‘Pitch trim’’ 	airplane, disagree by more than 200 feet for more

commands move the horizontal stabilizer. Pilots use 	than 5 seconds. 	7 Ethiopian Aircraft Accident Investigation

pitch trim to adjust the position of the horizontal 	6 Flight data recorder (FDR) data from the Lion 	Preliminary Report AI–01/19, dated March 2019,

stabilizer to achieve the desired flight path and to 	Air Flight 610 accident airplane indicated that on 	and the Ethiopian Interim Investigation Report of

manage the forces necessary to keep the airplane in 	the flight just prior to the accident flight (Lion Air 	accident MAX–8 ET–AVJ, ET–302, dated March

stable flight. 	Flight 043), the airplane experienced the same 	2020, can be found in the AD docket.

4 The angle of attack (or AOA) is the angle at 	single erroneously high AOA sensor failure 	8 MCAS is a function of the Speed Trim System

which the airplane wing meets the oncoming air. 	condition upon takeoff that the Lion Air Flight 610 	(STS), which is part of the airplane’s flight control

On the current 737 MAX, AOA is measured by two 	crew encountered. The flightcrew on Lion Air 	system. The STS provides automatic trim inputs to

independent AOA sensors, which are small vanes 	Flight 043 was able to maintain continued safe 	the horizontal stabilizer during manual flight. The

mounted on either side of the forward exterior of 	flight and land at their planned destination airport 	STS uses data from a variety of sources, such as

the fuselage. For the purposes of this NPRM, ‘‘high’’ 	in Jakarta. The flightcrew on Lion Air Flight 043 	pitot tubes and the AOA sensors, to calculate when

AOA is a relatively large angle (associated with 	had no prior awareness of this type of failure or 	to make commands. MCAS is activated only during

flight conditions outside of the normal flight 	how to respond to it. The FAA’s review of these 	manual flight, with flaps up, and when the AOA

envelope), and ‘‘low’’ AOA is a relatively small 	flights and associated risk assessments provided the 	sensors detect that the airplane is flying with a high

angle (associated with flight conditions within the 	basis for the revised pilot procedures contained in 	AOA, such as when climbing aggressively or

normal flight envelope). Although wing lift 	the interim action of the FAA’s emergency AD; 	performing excessively tight turns with high bank

jbell on DSKJLSW7X2PROD with PROPOSALS 
	increases with increased AOA, an excessively high 	specifically, the rationale was that if pilots were 	angles. MCAS makes pitch trim commands to the

airplane nose-up AOA can be hazardous, since 	provided awareness of the airplane and flightdeck 	horizontal stabilizer during a high AOA event so

eventually lift can be lost, causing the airplane to 	effects of this specific failure scenario and were 	that the 737 MAX handling qualities are compliant

stall. A stall occurs when the airflow around the 	provided appropriate instructions via the 	with FAA regulations (including 14 CFR 25.173).

wing is sufficiently disrupted to cause the wing to 	emergency AD, this would enable appropriate pilot 	9 An AOA disagree alert, or ‘‘AOA DISAGREE’’ on

no longer generate lift. To warn of an impending 	response to the erroneously high AOA failure 	the 737 MAX, is a visual alert on the airplane’s

stall, the 737 MAX is equipped with a ‘‘stick 	scenario for the period of time needed to fully 	PFDs that alerts the flightcrew of a disagreement

shaker,’’ which vibrates the control column, 	eliminate this unsafe condition with a software 	between the angles of attack measured by each of

providing tactile annunciation to the pilot. 	revision to the flight control computers. 	the airplane’s two AOA sensors.

VerDate Sep<11>2014 	16:21 Aug 05, 2020 	Jkt 250001 	PO 00000 	Frm 00002 	Fmt 4702 	Sfmt 4702 	E:\FR\FM\06AUP1.SGM 	06AUP1

47700 	Federal Register / Vol. 85, No. 152 / Thursday, August 6, 2020 / Proposed Rules

require operators to conduct an AOA 	command to move the horizontal 		Also, as a result of the installation of sensor system test and perform an 	stabilizer, such that the final horizontal 	this revised MDS software, operators operational readiness flight prior to 	stabilizer position (after the MCAS 
	would be required to remove ‘‘INOP’’ returning each airplane to service. 	command) would preserve the 	markers, if present, from the electronic Finally, operators with an existing FAA- 	flightcrew’s ability to control the 	flight instrument system (EFIS) panel of approved MEL would be required to 
	airplane pitch by using only the control 	the airplane, because the markers would incorporate more restrictive provisions 	column. The original design allowed 	no longer be necessary, due to other to dispatch the airplane with certain 	MCAS commands to be made without 	changes in the updated MDS software inoperative equipment. The new master 	consideration of the horizontal stabilizer 	that are unrelated to this unsafe minimum equipment list (MMEL), 	position—before or after the MCAS 	condition. These markers, labeled approved by the FAA, was published on 	command. 
	‘‘INOP,’’ indicate that one of the April 10, 2020, after undergoing a 		An undesired MCAS activation could 	positions on the dial that selects display public notice and comment process. 	prompt the flightcrew to perform a non- 	settings is inoperative.

Proposed Design Changes 	normal procedure. To ensure that after 	any foreseeable failure of the stabilizer 	recognize and respond to undesired 			To facilitate the flightcrew’s ability to

The FAA proposes mandating the 	system, safe flight is not dependent on 	horizontal stabilizer movement and the following changes to the 737 MAX type 	the timeliness of the flightcrew 	effects of a potential AOA sensor failure, design, to address the various aspects of 	performing a non-normal procedure, the 
	the FAA proposes to mandate revising the unsafe condition. 	FAA proposes multiple changes. 	and adding certain operating procedures 	To ensure that an erroneous signal 		First, as previously discussed, the 	(checklists) of the AFM14used by the from a failed single AOA sensor does 
	flight control laws would be changed to 	flightcrew for the 737 MAX. All not prevent continued safe flight and 	instead use inputs from two AOA 	transport category airplanes have non- landing, and specifically that it does not 		sensors for MCAS activation, so that 	normal checklists to aid the pilots in generate erroneous MCAS activation, 		there would not be an undesired MCAS 	responding to airplane failures. the FAA proposes to require installation 		activation due to a single AOA sensor 		The following is a general description of updated FCC software with revised 		failure that could lead a flightcrew to 
	of the changes that would be made to flight control laws10associated with 		perform a non-normal procedure. 	these checklists,15and the purpose of MCAS. These revised flight control laws 			Second, in the event that MCAS is 	each change. The FAA will conduct an would use inputs from both AOA 
		activated as intended (i.e., during a high 	operational evaluation before finalizing sensors to activate MCAS. This is in 		AOA event), the updated flight control 	these checklists. (See Flightcrew contrast to the original MCAS design, 		laws software would limit the number 	Training section in this preamble for which relied on data from only one 		of MCAS activations to one per high 	further information.) 
sensor at a time, and allowed repeated 				To reduce the workload on the 		AOA event, and limit the magnitude of 
MCAS activation as a result of input 			flightcrew when they suspect that the 		any single activation so that the 
from a single AOA sensor. 			airspeed indications are unreliable, the 	The updated FCC software would also 		flightcrew could maintain pitch control 	FAA proposes to revise the Airspeed compare the inputs from the two 		without needing to perform a non- 
	Unreliable checklist of the AFM. This sensors to detect a failed AOA sensor. 		normal procedure. 	checklist would be revised to (1) add a If the difference between the AOA 			The FAA also proposes requiring an 	step to allow the flightcrew to determine sensor inputs is above a calculated 
		additional software update that would 	a reliable airspeed indication without threshold,11the FCC would disable the 		alert the flightcrew to a disagreement 	the use of reference tables, (2) improve speed trim system (STS), including its 		between the two AOA sensors. This 	the procedure for go-arounds to allow MCAS function, for the remainder of 		disagreement indicates certain AOA 	for increased use of automation, (3) add that flight, and provide a corresponding 		sensor failures or a significant 	a step to ensure that erroneous altitude

indication of such deactivation on the 	calibration issue. The updated MDS 	information is not transmitted via the

flight deck. 		software would implement an AOA 	transponder to air traffic control (ATC), 	To ensure that MCAS will not 	DISAGREE alert on all 737 MAX 	and (4) add erroneous AOA as a command repeated movements of the 	airplanes. Some 737 MAX airplanes

horizontal stabilizer, the revised flight 	were delivered without this alert 	14 The AFM is an FAA-approved document that

control laws would permit only one 	feature, by error. While the lack of an 	manufacturers are required to furnish to owners

activation of MCAS per sensed high AOA event. A subsequent activation of 	AOA DISAGREE alert is not an unsafe 	condition itself, the FAA is proposing to 		upon delivery of the airplane, and that provides 		necessary safety information. See 14 CFR 25.1581. 		This information includes procedures (emergency MCAS would be possible only after the 	mandate this software update to restore 	and non-normal) for foreseeable but unusual

airplane returns to a low AOA state, 	compliance with 14 CFR 25.1301 and 	situations that necessitate flightcrew action. See 14

below the threshold that would cause MCAS activation. 		because the flightcrew procedures 		mandated by this AD now rely on this 			CFR 25.1585. These procedures provide the 			flightcrew with instructions, including checklists, 			on how to respond to these conditions. Some of 	The updated FCC software would also 	alert to guide flightcrew action. As a 	these conditions require immediate action by the

limit12the magnitude of any MCAS 	result of the changes proposed in this 	flightcrew, so some checklists identify certain tasks 			AD, differences between the two AOA 	that the flightcrew is expected to accomplish from 					memory; these items are commonly known as 		10 A flight control law generates commands to 	sensors greater than a certain 
	memory steps or ‘‘recall’’ items. Other conditions 	move flight control surfaces based on inputs from 	threshold13would cause an AOA 	have checklists that do not need to be memorized; 	the flightcrew and sensors on the airplane. Flight 	DISAGREE alert on the primary flight 	these items are commonly known as ‘‘reference’’ jbell on DSKJLSW7X2PROD with PROPOSALS 
	control laws reside in software, and are developed 	to generate commands from the flight control 	displays (PFDs). 	items. 						15 All of the checklists that the FAA proposes to 	computers that will achieve desired airplane 			revise or add to the AFM are already part of 	performance. 
	altitude and airspeed, and would be limited such 	Boeing’s Quick Reference Handbook, or QRH, for 		11 The calculated threshold would be a function 	that after the command is made, the pilot would be 	the 737 MAX (except for the IAS Disagree checklist, 	of the magnitude of the disagreement and the rate 	able to maintain level flight, climb, and descend, 	which is new to both the AFM and the QRH). The 	of change of the AOA sensor position values. 	using control column inputs only. 	QRH is a nonregulatory tool used by flightcrews 		12 The magnitude of the command varies 		13 More than 10 degrees difference for more than 	that includes information for non-normal and 	according to parameters such as the airplane’s 	10 seconds. 	emergency conditions, including AFM procedures.

VerDate Sep<11>2014 	16:21 Aug 05, 2020 	Jkt 250001 	PO 00000 	Frm 00003 	Fmt 4702 	Sfmt 4702 	E:\FR\FM\06AUP1.SGM 	06AUP1